# Switch Abstraction Interface

# Change Proposal

| Title | Host interface |
| --- | --- |
| Authors | Mellanox |
| Status | In Review |
| Type | Standards Track |
| Created | 3/9/2015 |
| SAI-Version | 0.9.2 |

Contents

List of Changes	i

1	Overview	1

1.1	Classification & Registration	1

1.1.1	Control traffic identification	1

1.1.2	Classification	2

1.1.3	Registration	3

1.2	Packet send and receive	3

1.2.1	Packet send and receive - net device	3

2	SAI object model and pipeline	4

2.1.1	SAI objects	4

2.2	Host if - Pipeline	12

2.2.1	Switch trap -> user channel port	12

2.2.2	Pre  L3 trap -> user channel L3	12

2.2.3	Local IP trap ->user channel L3	13

2.2.4	User define trap(router)->user channel FD	13

3	Specification	13

3.1	Host interface functionality	13

3.1.1	Host interface object	13

3.1.2	Host interface table entry	15

3.1.3	Host interface trap-id	18

3.1.4	Host interface trap group	22

3.1.5	Host interface packet send and receive	23

3.1.6	Host interface API	25

3.2	New Callback function for packet receive	25

3.3	Changes in saitypes.h	26

3.4	Changes in saiswitch.h	26

3.5	Configuration example – using netdev channel	26

3.5.1	Step 1- create three trap groups	26

3.5.2	Step 2- create host interface channel	27

3.5.3	Step 3- configure the trap-ids	28

3.5.4	Step 4- configure the host table	29

3.6	Configuration example – using file descriptor channel	30

3.6.1	Step 1- create three trap groups	30

3.6.2	Step 2- create host interface FD	30

3.6.3	Step 3- configure the trap-ids	30

3.6.4	Step 4- configure the host table	30

3.6.5	Step 5- send and receive	31

3.7	Configuration example – using packet receive callback channel	31

3.7.1	Step 1- create three trap groups	31

3.7.2	Step 2- configure the trap-ids	31

3.7.3	Step 3- configure the host table	32

3.7.4	Step 4- send and receive	32

3.8	Configuration example – trap via ACL	33

3.8.1	Step 1- configure the trap id	33

3.8.2	Step 2- create host interface netdev	33

3.8.3	Step 3- configuring the host table	33

3.8.4	Step 2- ACL configuration	33

# List of Changes

| Version | Changes | Name | Date |
| --- | --- | --- | --- |
| 0.9.2 | Base version |  | 2/5/2015 |
| 0.9.2 | Version 2 with review comments |  | 3/18/2015 |
| 0.9.2 | Version 3 with review comments |  | 4/7/2015 |
| 0.9.2 | Version 4 with review comments |  | 4/9/2015 |
| 0.9.5 | Version 9 with review comments |  | 10/27/2016 |

License

© 2014 Microsoft Corporation, Dell Inc., Facebook, Inc, Broadcom Corporation, Intel Corporation, Mellanox Technologies Ltd.

As of September 9, 2014, the following persons or entities have made this Specification available under the Open Web Foundation Final Specification Agreement (OWFa 1.0), which is available at http://www.openwebfoundation.org/legal/the-owf-1-0-agreements/owfa-1-0

Microsoft Corporation, Dell Inc., Facebook, Inc, Intel Corporation, Mellanox Technologies Ltd.

You can review the signed copies of the Open Web Foundation Agreement Version 1.0 for this Specification at http://opencompute.org/licensing/, which may also include additional parties to those listed above.

Your use of this Specification may be subject to other third party rights. THIS SPECIFICATION IS PROVIDED "AS IS." The contributors expressly disclaim any warranties (express, implied, or otherwise), including implied warranties of merchantability, noninfringement, fitness for a particular purpose, or title, related to the Specification. The entire risk as to implementing or otherwise using the Specification is assumed by the Specification implementer and user. IN NO EVENT WILL ANY PARTY BE LIABLE TO ANY OTHER PARTY FOR LOST PROFITS OR ANY FORM OF INDIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES OF ANY CHARACTER FROM ANY CAUSES OF ACTION OF ANY KIND WITH RESPECT TO THIS SPECIFICATION OR ITS GOVERNING AGREEMENT, WHETHER BASED ON BREACH OF CONTRACT, TORT (INCLUDING NEGLIGENCE), OR OTHERWISE, AND WHETHER OR NOT THE OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

THE FOLLOWING IS A LIST OF MERELY REFERENCED TECHNOLOGY: Microprocessor technology, semiconductor manufacturing technology, operating system technology (including without limitation networking operating system technology), emulation technology, graphics technology, video technology, integrated circuit packaging technology and the like, compiler technologies, object oriented technology, optical/RF communications technology including chip I/O and driver technology, bus technology, memory chip technology (including, without limitation, NAND memory, NOR memory, resistive RAM (RRAM), seek scan probe (SSP) memory, nonvolatile memory (including without limitation, memory based on chalcogenide materials, phase change memory (PCM), one or more stacked layers of memory cells, embedded PCM memories, non-volatile cache memory, solid state drives, SRAM, embedded DRAM, ferro-electric memory, and polymer memory)) and/or health-related and medical technology. IMPLEMENTATION OF THESE TECHNOLOGIES MAY BE SUBJECT TO THEIR OWN LEGAL TERMS.

# Overview

In the normal operation of a network, the network device CPU is required to handle packets. Most notably, control-plane traffic (System-to-System protocol coordination traffic - e.g. LACP, IGMP ARP … ) and device management traffic must reach the CPU. In addition, The CPU has limited resources and therefore can obviously handle only a limited amount of data. As a result, it is possible that the processing of some of the traffic being sent to the CPU may be delayed or, in extreme cases, some of the traffic may even be dropped.

A system which is properly configured ensures that high priority traffic will experience less (if at all) drops and will get to the CPU sooner than any other lower priority classified traffic. On the software side the application should ensure that high classified traffic will be prioritized in CPU processing time over other traffic.

In order to guarantee proper operation of the switch, the traffic types being sent to the CPU must be handled according to a predefined policy. Within this policy, the priorities, the bandwidth (allowed rate) and the burst of each traffic type should be defined.

Host interface SAI, composed from two parts

Classification & Registration

# SAI object model and pipeline

Router

Port

Bridge

ACL

User define

ACL trap

User define

Router  trap

Host interface  Group

CPU

Port

trap

-

id table

Switch  traps

Port

Port

Port

Host table

trap

-

id table

router  traps

Host

interface

Port

netdev

Host interface  Group

Host interface  Group

Host

interface

LAG

Host

interface

vlan

Host

interface

FD

Host

interface

Port

netdev

Host

interface

LAG

Host

Interface

LAG

Host

interface

vlan

Host

interface

vlan

Host

interface

FD

Host

interface

FD

Host Interface

Port

## SAI objects

### Host interface

SAI object "host interface" is an interface to application in order to be able to send and receive control packet from the HW

Host interface types:

CB- global call back function

FD –file descriptor

Port net device- net device that represent a physical port

LAG net device - net device that represent a logical port

L3 net device   - net device that represent a L3 interface

Generic netlink - Linux generic netlink interface

### Host table

Match action table object host table

Mux between control traffic to the corresponding   "host interface" object

Host interface table supports five type of entries

{Port,trap_id}(SAI_HOSTIF_TABLE_ENTRY_TYPE_PORT)

{LAG,trap_id}(SAI_HOSTIF_TABLE_ENTRY_TYPE_LAG )

{vlan,trap_id}(SAI_HOSTIF_TABLE_ENTRY_TYPE_VLAN)

{wildcard,trap_id}(SAI_HOSTIF_TABLE_ENTRY_TYPE_TRAP_ID)

{wildcard, wildcard}(SAI_HOSTIF_TABLE_ENTRY_TYPE_WILDCARD)

#### Host table action

Host table action {

{host_if_type,host_if_data}

}

Valid option are

{ host_if_type_CB ,NULL}- send the data using call back

{ host_if_type_FD ,host_if_object_id}- send the data using FD given in the host_if_object_id

{ host_if_type_net_phy_port ,NULL}- send the data using Linux netdev type port

{ host_if_type_net_logical_port ,NULL}- send the data using Linux netdev logical port {LAG or port}

{ host_if_type_net_L3 ,NULL}- send the data using Linux netdev type L3 interface

{ host_if_type_genericNetlink ,NULL}- send the data using Linux netdev type L3 interface

#### Net device selection algorithm

- Host Table  port Entry format

Match: {port},{trap-id}

Actions: { Host table action }

- Host Table LAG  Entry format

Match: {LAG},{trap-id}

Actions: { Host table action }

- Host Table VLAN Entry format

Match: {VLAN},{trap-id}

Actions: { Host table action }

- Host Table wildcard Entry format

Match: {*},{trap-id}

Actions: { Host table action }

Match: {*},{* }

Actions: { Host table action }

- Table default action: Call Back

#### Host interface trap id table

Match action table object trap-id table

Map between control traffic " trap-id" to pipeline action  & profile

Trap-id Table Entry format

Match: {port,*},{trap-id}

Actions: {drop, forward, copy to cpu } {trap_group_pointer}

Table default action: forward

Two instantiation of Trap-id Table exists is the SAI pipeline:

Switch trap table: pipeline location switch port - all l2 control entry will be inserted to this table

Router trap table: pipeline location between bridge and router all l3 control entry will be inserted to this table

### Host interface trap group

Profile object

Configure common attribute between set of trap-ids entries

Trap group attribute:

Priority – will be in use to resolve conflicts between contradiction actions

CPU Queue – CPU queue to use

Policer – rate limiter

### Control traffic identification trap-id

SAI assigns a unique identifier trap_id to each control traffic

There are different types of trap id:

L2 control protocol trap id:

Trap ids for well-known l2 control protocol, e.g. STP, LACP …

By default PDU will be dropped all other will be forward

Pre L3 control protocol trap id:

Trap ids for well-known L3 control protocol, e.g. ARP, OSPF…

By default all L3 control will be forward

Local IP (IP2ME) control protocol trap id:

Trap ids for well-known local L3 control protocol, e.g. BFD, BGP…

By default all IP2ME traffic will be drooped

Pipeline exception trap id:

Trap id for an exception in the switch-router forwarding pipeline, e.g. router TTL=1, RPF …

By default Pipeline exception traffic will be drooped

User define trap id:

Application has the ability to extend and define additional control traffic trap id in order to support a new or proprietary control protocol or to define an additional exception in the switch pipeline. This capability is achieved by providing the ability to define trap-id via the switch-router pipeline, e.g. ACL, router …

By default, user define CPU copy will be drooped (including Sflow)

### Trap action

Application can register to receive a trap_id. In addition, it can control the action to apply on the trap-id, options are:

Ignore – forward the trap-id as a regular packet

Trap – terminate pipeline and send packet to CPU

Mirror- sent a copy to the CPU, the original packet will continue the pipeline

Discard – drop

#### Trap actions

|  | L2 control protocol trap id | Pre L3 control protocol trap id | Local IP (IP2ME) control protocol trap id | Pipeline exception trap id | User define trap id |
| --- | --- | --- | --- | --- | --- |
| Ignore | Forward to bridge | Forward router | Trap to CPU as IP2ME | NA | NA |
| Trap | L2- trap and terminate pipeline | L2- forward L3 - trap and terminate pipeline | L2- forward L3 -forward IP2ME- trap | Trap according to the pipeline stage | NA |
| Mirror | L2- trap and Forward to bridge | L2- forward L3 - Forward to Router | NA | NA | NA |
| Discard | L2- discard | L2- forward L3 – discard | L2- forward L3 -forward IP2ME- discard | Discard according to the pipeline stage | NA |

### Packet send and receive

SAI provides three different channels in order to send and receive packet from and to the CPU

- OS network device

- a generic file socket

- send function and receive function / receive function callback

### Packet send and receive - net device

SAI will provide the ability to receive (and send) trap-id on the operation system network device infrastructure.

This is done in order to enable standard application such as Quagga to operate the switch as if it is a host.

SAI will provide the ability to create these net devices whether they represent a physical port or a L3 router interface.

On RX, the driver should demux the packet to the relevant net device according to incoming port / RIF ID.

On TX, the application chooses which net device to use

# Host if - Pipeline

## L2/port trap -> user channel port netDev

### Pre L3 trap -> user channel L3 netDev

### IP2ME trap ->user channel L3

### User define trap(router)->user channel FD

# Specification

[This section describes an overview of the proposed interface/API.]

## Host interface functionality

### Host interface object

/*

*  Attribute data for SAI_HOST_INTERFACE_ATTR_TYPE

*/

typedef enum _sai_host_interface_type_t

{

SAI_HOST_INTERFACE_TYPE_NETDEV,

/* file descriptor */

SAI_HOST_INTERFACE_TYPE_FD

} sai_host_interface_type_t;

/*

*  Host interface attribute IDs

*/

typedef enum _sai_host_interface_attr_t

{

/* READ-ONLY */

/* READ-WRITE */

/* Type [sai_host_interface_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_HOST_INTERFACE_ATTR_TYPE,

/* host intarfce object-id

* Valid only when SAI_HOST_INTERFACE_ATTR_TYPE == SAI_HOST_INTERFACE_TYPE_NETDEV

* port netdev will be created when the object type is  port

*LAG netdev will be created when the object type is  LAG

*VLAN netdev will be created when the object type is  VLAN ID

* (MANDATORY_ON_CREATE | CREATE_ONLY  when SAI_HOST_INTERFACE_ATTR_TYPE ==

* SAI_HOST_INTERFACE_TYPE_NETDEV */

SAI_HOST_INTERFACE_ATTR_OBJ_ID,

/* Name [char[HOST_INTERFACE_NAME_SIZE]]

* The maximum number of charactars for the name is HOST_INTERFACE_NAME_SIZE - 1 since

* it needs the terminating null byte ('\0') at the end.

* Valid only when SAI_HOST_INTERFACE_ATTR_TYPE == SAI_HOST_INTERFACE_TYPE_NETDEV

* (MANDATORY_ON_CREATE when SAI_HOST_INTERFACE_ATTR_TYPE ==

* SAI_HOST_INTERFACE_TYPE_NETDEV */

SAI_HOST_INTERFACE_ATTR_NAME,

/* Custom range base value */

SAI_HOST_INTERFACE_ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_host_interface_attr_t;

/*

* Routine Description:

*    Create host interface

*

* Arguments:

*    [out] hif_id - host interface id

*    [in] attr_count - number of attributes

*    [in] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t(*sai_create_host_interface_fn)(

_Out_ sai_object_id_t * hif_id,

_In_ uint32_t attr_count,

_In_ sai_attribute_t *attr_list

);

/*

* Routine Description:

*    Remove host interface

*

* Arguments:

*    [in] hif_id - host interface id

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t(*sai_remove_host_interface_fn)(

_In_ sai_object_id_t hif_id

);

/*

* Routine Description:

*    Set host interface attribute

*

* Arguments:

*    [in] hif_id - host interface id

*    [in] attr - attribute

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_set_host_interface_attribute_fn)(

_In_ sai_object_id_t hif_id,

_In_ const sai_attribute_t *attr

);

/*

* Routine Description:

*    Get host interface attribute

*

* Arguments:

*    [in] hif_id - host interface id

*    [in] attr_count - number of attributes

*    [inout] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_get_host_interface_attribute_fn)(

_In_ sai_object_id_t  hif_id,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Host interface table entry

/*

*  Attribute data for SAI_HOST_INTERFACE_TABLE_ENTRY

*/

typedef enum _sai_host_interface_table_entry_type_t

{

/* Port-based Host Interface entry  Type */

SAI_HOST_INTERFACE_ENTRY_TYPE_PORT,

/* LAG based Host Interface entry Type */

SAI_HOST_INTERFACE_ENTRY_TYPE_LAG,

/* vlan based Host Interface entry Type */

SAI_HOST_INTERFACE_ENTRY_TYPE_VLAN,

/* wildcard Interface entry Type */

SAI_HOST_INTERFACE_ENTRY_TYPE_TRAP_ID,

/* wildcard Interface , wildcard trap id */

SAI_HOST_INTERFACE_ENTRY_TYPE_WILDCARD,

} sai_host_interface_table_entry_type_t;

/**

* @brief Attribute data for SAI_HOST_INTERFACE_TABLE_ENTRY_ATTR_CHANNEL_TYPE

*/

typedef enum _sai_host_interface_table_entry_channel_type_t

{

/** receive packets via callback */

SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_CB,

/** receive packets via file descriptor */

SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_FD,

/** receive packets via Linux netdev type port */

SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_NETDEV_PHYSICAL_PORT,

/** receive packets via Linux netdev logical port (LAG or port) */

SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_NETDEV_LOGICAL_PORT,

/** receive packets via Linux netdev L3 interface */

SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_NETDEV_L3

} sai_host_interface_table_entry_channel_type_t;

/*

*  Attribute data for SAI_HOST_INTERFACE_TABLE_ENTRY

*/

typedef enum _sai_host_interface_table_user_channel

{

/* for user channel callback */

SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_CB,

/* for user channel file descriptor  */

SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_FD,

/* for user channel physical port */

SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_NET_PHY_PORT,

/* for user channel logical port  */

SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_NET_LOGICAL_PORT,

/* for user channel L3 intarface */

SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_NET_L3,

} sai_host_interface_table_user_channel_t;

/*

*  Host interface table entry attribute IDs

*/

typedef enum _sai_host_interface_table_entry_attr_t

{

/* READ-ONLY */

/* READ-WRITE */

/* Type [sai_host_interface_table_entry_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_HOST_INTERFACE_TABLE_ENTRY_ATTR_TYPE,

/* host intarfce table entry match field object-id

* Valid only when SAI_HOST_INTERFACE_ATTR_TYPE == SAI_HOST_INTERFACE_ENTRY_TYPE_PORT

* || SAI_HOST_INTERFACE_ENTRY_TYPE_LAG || SAI_HOST_INTERFACE_ENTRY_TYPE_VLAN

* should be port object when type is  SAI_HOST_INTERFACE_ENTRY_TYPE_PORT

* should be lag  object when type is SAI_HOST_INTERFACE_ENTRY_TYPE_LAG

* should be VLAN ID  object when type is SAI_HOST_INTERFACE_ENTRY_TYPE_VLAN

* (MANDATORY_ON_CREATE | CREATE_ONLY)  */

SAI_HOST_INTERFACE_TABLE_ENTRY_FIELD_OBJ_ID,

/* not Valid only when SAI_HOST_INTERFACE_ATTR_TYPE == SAI_HOST_INTERFACE_ENTRY_TYPE_WILDCARD  host intarfce table entry match field trap-id []

* (MANDATORY_ON_CREATE | CREATE_ONLY)  */

SAI_HOST_INTERFACE_TABLE_ENTRY_FIELD_TRAP_ID,

/*host intarfce table entry action channel  [sai_host_interface_table_entry_channel_type_t]

Valid only when SAI_HOST_INTERFACE_TABLE_ENTRY_ACTION_USER_CHANNEL_TYPE == SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_FD

* (MANDATORY_ON_CREATE | CREATE_ONLY)  */

SAI_HOST_INTERFACE_TABLE_ENTRY_ACTION_CHANNEL,

/*host intarfce table entry action user channel type [sai_host_interface_table_user_channel_t]

* (MANDATORY_ON_CREATE | CREATE_ONLY)  */

SAI_HOST_INTERFACE_TABLE_ENTRY_ACTION_USER_CHANNEL_TYPE,

/*host intarfce table entry action target host intarface  [sai_object_id_t]

Valid only when SAI_HOST_INTERFACE_TABLE_ENTRY_ACTION_USER_CHANNEL_TYPE == SAI_HOST_INTERFACE_ENTRY_USER_CHANNEL_FD

* (MANDATORY_ON_CREATE | CREATE_ONLY)

* Valid only when SAI_HOST_INTERFACE_TABLE_ENTRY_ACTION_CHANNEL=

* SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_FD

*/

SAI_HOST_INTERFACE_TABLE_ENTRY_ACTION_HOSTIF_ID,

/* Custom range base value */

SAI_HOST_INTERFACE_TABLE_ENTRY ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_host_interface_table_entry_attr_t;

/*

* Routine Description:

*    Create host interface table entry

*

* Arguments:

*    [out] hif_table_entry - host interface table entry

*    [in] attr_count - number of attributes

*    [in] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t(*sai_create_host_interface_table_entry_fn)(

_Out_ sai_object_id_t * hif_table_entry,

_In_ uint32_t attr_count,

_In_ sai_attribute_t *attr_list

);

/*

* Routine Description:

*    Remove host interface table entry

*

* Arguments:

*    [in] hif_table_entry - host interface table entry

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t(*sai_remove_host_interface_table_entry_fn)(

_In_ sai_object_id_t hif_table_entry

);

/*

* Routine Description:

*    Set host host interface table entry attribute

*

* Arguments:

*    [in] hif_table_entry - host interface table entry

*    [in] attr - attribute

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_set_host_interface_table_entry_attribute_fn)(

_In_ sai_object_id_t hif_table_entry,

_In_ const sai_attribute_t *attr

);

/*

* Routine Description:

*    Get host host interface table entry attribute

*

* Arguments:

*    [in] hif_table_entry - host interface table entry

*    [in] attr_count - number of attributes

*    [inout] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_get_host_interface_table_entry_attribute_fn)(

_In_ sai_object_id_t  hif_table_entry,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Host interface trap-id

typedef enum _sai_hostif_trap_id_t

{

/*

* control protocol

*/

/*

* switch trap

*/

SAI_HOSTIF_TRAP_ID_STP,

SAI_HOSTIF_TRAP_ID_LACP,

SAI_HOSTIF_TRAP_ID_EAPOL,

SAI_HOSTIF_TRAP_ID_LLDP,

SAI_HOSTIF_TRAP_ID_PVRST,

SAI_HOSTIF_TRAP_ID_SWITCH_CUSTOM_RANGE_BASE  = 0x10000000,

/*

* router trap

*/

SAI_HOSTIF_TRAP_ID_ARP_REQUEST  = 0x20000000,

SAI_HOSTIF_TRAP_ID_ARP_RESPONSE,

SAI_HOSTIF_TRAP_ID_DHCP,

SAI_HOSTIF_TRAP_ID_OSPF,

SAI_HOSTIF_TRAP_ID_PIM,

SAI_HOSTIF_TRAP_ID_VRRP,

SAI_HOSTIF_TRAP_ID_DHCPV6,

SAI_HOSTIF_TRAP_ID_OSPFV6,

SAI_HOSTIF_TRAP_ID_VRRPV6,

SAI_HOSTIF_TRAP_ID_IPV6_NEIGHBOR_DISCOVERY,

SAI_HOSTIF_TRAP_ID_IGMP_TYPE_QUERY,

SAI_HOSTIF_TRAP_ID_IGMP_TYPE_LEAVE,

SAI_HOSTIF_TRAP_ID_IGMP_TYPE_V1_REPORT,

SAI_HOSTIF_TRAP_ID_IGMP_TYPE_V2_REPORT,

SAI_HOSTIF_TRAP_ID_IGMP_TYPE_V3_REPORT,

SAI_HOSTIF_TRAP_ID_IPV6_MLD_V1_V2 ,

SAI_HOSTIF_TRAP_ID_IPV6_MLD_V1_REPORT,

SAI_HOSTIF_TRAP_ID_IPV6_MLD_V1_DONE,

SAI_HOSTIF_TRAP_ID_MLD_V2_REPORT ,

SAI_HOSTIF_TRAP_ID_ROUTER_CUSTOM_RANGE_BASE  = 0x30000000,

/*

* local IP traps

*/

SAI_HOSTIF_TRAP_ID_BGP,

SAI_HOSTIF_TRAP_ID_IP2ME,

SAI_HOSTIF_TRAP_ID_SSH,

SAI_HOSTIF_TRAP_ID_SNMP,

SAI_HOSTIF_TRAP_ID_BGPV6,

SAI_HOSTIF_TRAP_ID_CUSTOM_LOCAL_IP_RANGE_BASE  = 0x40000000

/*

* router exceptions

*/

SAI_HOSTIF_TRAP_ID_L3_MTU_ERROR  = 0x40000000,

SAI_HOSTIF_TRAP_ID_TTL_ERROR,

SAI_HOSTIF_TRAP_ID_L3_RPF,

SAI_HOSTIF_TRAP_ID_L3_ASSERT,

SAI_HOSTIF_TRAP_ID_CUSTOM_EXCEPTION_RANGE_BASE  = 0x50000000,

/*

* user define

*/

SAI_HOSTIF_TRAP_ID_ROUTER_MIN,

SAI_HOSTIF_TRAP_ID_ROUTER_MAX=SAI_HOSTIF_TRAP_ID_ROUTER_MIN+SAI_HOSTIF_TRAP_ID_TABLE_RANGE,

SAI_HOSTIF_TRAP_ID_NEIGH_MIN,

SAI_HOSTIF_TRAP_ID_FDB_MAX=SAI_HOSTIF_TRAP_ID_NEIGH_MIN+SAI_HOSTIF_TRAP_ID_TABLE_RANGE,

SAI_HOSTIF_TRAP_ID_ACL_MIN,

SAI_HOSTIF_TRAP_ID_ACL_MAX=SAI_HOSTIF_TRAP_ID_ACL_MIN+SAI_HOSTIF_TRAP_ID_TABLE_RANGE,

SAI_HOSTIF_TRAP_ID_CUSTOM_RANGE_BASE  = 0x60000000,

} sai_hostif_trap_id_t;

typedef enum _sai_hostif_trap_attr_t

{

/* exclude specific ports [sai_port_list_t] defult is empty /

SAI_HOSTIF_TRAP_ATTR_EXCLUDE_PORT_LIST,

/* trap action [sai_packet_action_t] (MANDATORY_ON_CREATE) */

SAI_HOSTIF_TRAP_ATTR_TRAP_ACTION,

/* trap-group ID for the trap [sai_object_id_t] (MANDATORY_ON_CREATE) */                                     SAI_HOSTIF_TRAP_ATTR_TRAP_GROUP,

/* trap priority [uint32_t] (default to 0). This is equivalent to

* ACL entry priority SAI_ACL_ENTRY_ATTR_PRIORITY */

SAI_HOSTIF_TRAP_ATTR_TRAP_PRIORITY,

SAI_HOSTIF_TRAP_ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_hostif_trap_attr_t;

/*

* Routine Description:

*    Create host interface trap

*

* Arguments:

*  [in] hostif_trap_id - host interface trap id

*  [in] attr_count - number of attributes

*  [in] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t(*sai_create_hostif_trap_fn)(

_In_ sai_hostif_trap_id_t hostif_trapid,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/*

* Routine Description:

*    Remove host interface trap

*

* Arguments:

*  [in] hostif_trap_id - host interface trap id

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t(*sai_remove_hostif_trap_fn)(

_In_ sai_hostif_trap_id_t hostif_trapid

);

/*

* Routine Description:

*   Set trap attribute value.

*

* Arguments:

*    [in] hostif_trap_id - host interface trap id

*    [in] attr - attribute

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_set_hostif_trap_attribute_fn )(

_In_ sai_hostif_trap_id_t hostif_trapid,

_In_ sai_attribute_t *attr

);

/*

* Routine Description:

*   Get trap attribute value.

*

* Arguments:

*    [in] hostif_trap_id - host interface trap id

*    [in] attr_count - number of attributes

*    [in,out] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_get_hostif_trap_attribute_fn )(

_In_ sai_hostif_trap_id_t hostif_trapid,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Host interface trap group

typedef enum _sai_hostif_trap_group_attr_t

{

/* Admin Mode [bool] (default to TRUE) */

SAI_HOSTIF_TRAP_GROUP_ATTR_ADMIN_STATE,

/* group priority [uint32_t] (MANDATORY_ON_CREATE). This is equivalent

* to ACL table priority SAI_ACL_TABLE_ATTR_PRIORITY */

SAI_HOSTIF_TRAP_GROUP_ATTR_PRIO,

/* group egress queue [uint32_t] (MANDATORY_ON_CREATE) */

SAI_HOSTIF_TRAP_GROUP_ATTR_QUEUE,

/* sai_object_id_t [uint32_t] */

SAI_HOSTIF_TRAP_GROUP_ATTR_POLICER,

SAI_HOSTIF_TRAP_GROUP_ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_hostif_trap_group_attr_t;

/*

* Routine Description:

*    Create host interface trap group

*

* Arguments:

*  [out] hostif_group_id  - host interface trap group id

*  [in] attr_count - number of attributes

*  [in] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_create_hostif_trap_group_fn)(

_Out_ sai_object_id_t *hostif_trap_group_id,

_In_ uint32_t attr_count,

_In_ sai_attribute_t * attr_list

);

/*

* Routine Description:

*    Remove host interface trap group

*

* Arguments:

*  [in] hostif_trap_group_id -host interface trap group id

*

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_remove_hostif_trap_group_fn)(

_In_ sai_object_id_t hostif_trap_group_id

);

/*

* Routine Description:

*   Set host interface trap group attribute value.

*

* Arguments:

*    [in] hostif_trap_group_id - host interface trap group id

*    [in] attr - attribute

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_set_hostif_trap_group_attribute_fn )

(

_In_ sai_object_id_t hostif_trap_group_id,

_In_ sai_attribute_t *attr

);

/*

* Routine Description:

*   get host interface trap group attribute value.

*

* Arguments:

*    [in] hostif_trap_group_id - host interface trap group id

*    [in] attr_count - number of attributes

*    [in,out] attr_list - array of attributes

*

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (*sai_get_hostif_trap_group_attribute_fn )(

_In_ sai_object_id_t hostif_trap_group_id,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Host interface packet send and receive

typedef enum _sai_tx_type

{

SAI_TX_TYPE_PIPELINE_LOOKUP,

SAI_TX_TYPE_PIPELINE_BYPASS,

SAI_TX_TYPE_CUSTOM_RANGE_BASE  = 0x10000000

} sai_tx_type_t;

typedef enum _sai_packet_attr

{

/* Trap ID [sai_hostif_trap_id_t] */

SAI_PACKET_TRAP_ID,

/* Ingress port [sai_object_id_t] */

SAI_PACKET_INGRESS_PORT,

/* Ingress LAG [sai_object_id_t] */

SAI_PACKET_INGRESS_LAG,

/* Egress port [sai_object_id_t].

* Either Port, or LAG, or both attributes, should be provided on TX */

SAI_PACKET_EGRESS_PORT,

/* Egress LAG [sai_object_id_t] */

SAI_PACKET_EGRESS_LAG,

/* Egress Queue [sai_object_id_t]. Optional */

SAI_PACKET_EGRESS_QUEUE,

/* TX type [sai_tx_type_t]. Mandatory */

SAI_PACKET_TX_TYPE,

} sai_packet_attr_t;

/*

* Routine Description:

*   hostif receive function

*

* Arguments:

*    [in]  hif_id  - host interface id

*    [out] buffer – packet buffer

*    [in,out] buffer_size- [in] allocated buffer size. [out] actual packet size in bytes

*    [in,out] attr_count – [in] allocated list size. [out] number of attributes

*    [out] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    SAI_STATUS_BUFFER_OVERFLOW if buffer_size is insufficient,

*    and buffer_size will be filled with required size. Or

*    if attr_count is insufficient, and attr_count

*    will be filled with required count.

*    Failure status code on error

*/

typedef sai_status_t (* sai_recv_hostif_packet_fn)(

_In_ sai_object_id_t  hif_id,

_Out_ void *buffer,

_Inout_ sai_size_t *buffer_size,

_Inout_ uint32_t *attr_count,

_Out_ sai_attribute_t *attr_list

);

/*

* Routine Description:

*   hostif send function

*

* Arguments:

*    [in] hif_id  - host interface id. only valid for send through FD channel. 0 for send

*    through CB channel.

*    [In] buffer – packet buffer

*    [in] buffer size- packet size in bytes

*    [in] attr_count - number of attributes

*    [in] attr_list - array of attributes

*

* Return Values:

*    SAI_STATUS_SUCCESS on success

*    Failure status code on error

*/

typedef sai_status_t (* sai_send_hostif_packet_fn)(

_In_ sai_object_id_t  hif_id,

_Inout_ void *buffer,

_Inout_ sai_size_t *buffer_size,

_In_ uint32_t attr_count,

_In_ sai_attribute_t *attr_list

);

### Host interface API

/*

* hostif methods table retrieved with sai_api_query()

*/

typedef struct _sai_host_interface_api_t

{

sai_create_host_interface_fn                   create_host_interface;

sai_remove_host_interface_fn                   remove_host_interface;

sai_set_host_interface_attribute_fn            set_host_interface_attribute;

sai_get_host_interface_attribute_fn            get_host_interface_attribute;

sai_create_host_interface_table_entry_fn       create_host_interface_table_entry;

sai_remove_host_interface_table_entry_fn       remove_host_interface_table_entry;

sai_set_host_interface_table_entry attribute_fn set_host_interface_table_entry attribute;

sai_get_host_interface_table_entry attribute_fn get_host_interface_table_entry attribute;

sai_create_hostif_trap_fn                      create_hostif_trap;

sai_remove_hostif_trap_fn   	               remove_hostif_trap;

sai_set_hostif_trap_attribute_fn               set_trap_attribute;

sai_get_hostif_trap_attribute_fn               get_trap_attribute;

sai_create_hostif_trap_group_fn                create_hostif_trap_group;

sai_remove_hostif_trap_group_fn   	       remove_hostif_trap_group;

sai_set_hostif_trap_group_attribute_fn         set_trap_group_attribute;

sai_get_hostif_trap_group_attribute_fn         get_trap_group_attribute;

sai_recv_hostif_packet_fn                      recv_packet;

sai_send_hostif_packet_fn                      send_packet;

} sai_host_interface_api_t;

## New Callback function for packet receive

/* * Switch notification table passed to the adapter via sai_initialize_switch()

*/

typedef struct _sai_switch_notification_t

{

sai_switch_state_change_notification_fn on_switch_state_change;

sai_fdb_event_notification_fn on_fdb_event; sai_packet_event_notification_fn on_packet_event;

sai_port_state_change_notification_fn on_port_state_change;

sai_switch_shutdown_request_fn on_switch_shutdown_request;

} sai_switch_notification_t; typedef void (*sai_packet_event_notification_fn)( _In_ const void *buffer, _In_ sai_size_t buffer_size, _In_ uint32_t attr_count, _In_ const sai_attribute_t *attr_list );

## Changes in saitypes.h

typedef size_t sai_size_t;

## Changes in saiswitch.h

-    /* Action for Packets with TTL 0 or 1 [sai_packet_action_t]

-       (default to SAI_PACKET_ACTION_TRAP) */

-    SAI_SWITCH_ATTR_VIOLATION_TTL1_ACTION,

## Configuration example – using netdev channel

In the below example we will configure a system running L2 {STP, LACP, IGMP snooping}, L3 {BGP}.

We will create 3 QoS groups for the traffic to CPU

High – for control STP, LACP, BGP

Medium – IGMP packet, ARP

Low – L3 exception (non-resolved ARP)

The system has 4 physical ports, and the method for send and receive of CPU traffic is via OS net device

### Step 1- create three trap groups

sai_object_id_t low_prio_group;

sai_object_id_t med_prio_group;

sai_object_id_t high_prio_group;

sai_attribute_t sai_attr_list[3];

sai_attr_list[0].id=SAI_HOSTIF_TRAP_GROUP_ATTR_PRIO;

sai_attr_list[0].value=7;

sai_attr_list[1].id=SAI_HOSTIF_TRAP_GROUP_ATTR_QUEUE;

sai_attr_list[1].value=high_queue_id; // high_queue_id is a queue element created via QoS SAI API

sai_attr_list[2].id= SAI_HOSTIF_TRAP_GROUP_ATTR_POLICER;

sai_attr_list[2].value=high_policer_id; //high_policer_id is a policer element created via policer  SAI API

sai_create_hostif_trap_group_fn(&high_prio_group, 3, sai_attr_list);

sai_attr_list[0].id=SAI_HOSTIF_TRAP_GROUP_ATTR_PRIO;

sai_attr_list[0].value=6;

sai_attr_list[1].id= SAI_HOSTIF_TRAP_GROUP_ATTR_QUEUE;

sai_attr_list[1].value=med_queue_id; // high_queue_id is a queue element created via QoS SAI API

sai_attr_list[2].id= SAI_HOSTIF_TRAP_GROUP_ATTR_POLICER;

sai_attr_list[2].value=med_policer_id; //high_policer_id is a policer element created via policer  SAI API

sai_create_hostif_trap_group_fn(&med_prio_group,3, sai_attr_list);

sai_attr_list[0].id=SAI_HOSTIF_TRAP_GROUP_ATTR_PRIO;

sai_attr_list[0].value=5;

sai_attr_list[1].id= SAI_HOSTIF_TRAP_GROUP_ATTR_QUEUE;

sai_attr_list[1].value=low_queue_id; // high_queue_id is a queue element created via QoS SAI API

sai_attr_list[2].id= SAI_HOSTIF_TRAP_GROUP_ATTR_POLICER;

sai_attr_list[2].value=low_policer_id; //high_policer_id is a policer element created via policer  SAI API

sai_create_hostif_trap_group_fn(&low_prio_group,3, sai_attr_list);

### Step 2- create host interface channel

Creation of port net devices in the OS

sai_object_id_t host_if_id[4];

sai_attribute_t sai_if_channel_attr[3];

sai_if_channel_attr[0].id=SAI_HOST_INTERFACE_ATTR_TYPE;

sai_if_channel_attr[0].value=SAI_HOST_INTERFACE_TYPE_NETDEV;

sai_if_channel_attr[1].id= SAI_HOST_INTERFACE_TYPE_OBJ_ID;

sai_if_channel_attr[1].value=port_id[0]; // port_id[X] is a port element created via port SAI API

sai_if_channel_attr[2].id= SAI_HOST_INTERFACE_ATTR_NAME;

sai_if_channel_attr[2].value=”port1”;

sai_create_host_interface_fn(&host_if_id[0], 3, sai_if_channel_attr);

sai_if_channel_attr[0].id= SAI_HOST_INTERFACE_ATTR_TYPE;

sai_if_channel_attr[0].value=SAI_HOST_INTERFACE_TYPE_NETDEV;

sai_if_channel_attr[1].id= SAI_HOST_INTERFACE_TYPE_OBJ_ID;

sai_if_channel_attr[1].value=port_id[1]; // port_id[X] is a port element created via port SAI API

sai_if_channel_attr[2].id= SAI_HOST_INTERFACE_ATTR_NAME;

sai_if_channel_attr[2].value=”port2”;

sai_create_host_interface_fn(&host_if_id[1] ,3, sai_if_channel_attr);

sai_if_channel_attr[0].id= SAI_HOST_INTERFACE_ATTR_TYPE;

sai_if_channel_attr[0].value=SAI_HOST_INTERFACE_TYPE_NETDEV;

sai_if_channel_attr[1].id= SAI_HOST_INTERFACE_TYPE_OBJ_ID;

sai_if_channel_attr[1].value=port_id[2]; // port_id[X] is a port element created via port SAI API

sai_if_channel_attr[2].id= SAI_HOST_INTERFACE_ATTR_NAME;

sai_if_channel_attr[2].value=”port3”;

sai_create_host_interface_fn(&host_if_id[2] ,3, sai_if_channel_attr);

sai_if_channel_attr[0].id= SAI_HOST_INTERFACE_ATTR_TYPE;

sai_if_channel_attr[0].value=SAI_HOST_INTERFACE_TYPE_NETDEV;

sai_if_channel_attr[1].id= SAI_HOST_INTERFACE_TYPE_OBJ_ID;

sai_if_channel_attr[1].value=port_id[3]; // port_id[X] is a port element created via port SAI API

sai_if_channel_attr[2].id= SAI_HOST_INTERFACE_ATTR_NAME;

sai_if_channel_attr[2].value=”port4”;

sai_create_host_interface_fn(&host_if_id[3], 3, sai_if_channel_attr);

### Step 3- configure the trap-ids

// configure STP trap-id

sai_attribute_t sai_trap_attr[3];

sai_object_id_t host_trap_id[4];

sai_trap_attr[0].id=SAI_HOSTIF_TRAP_ATTR_TRAP_GROUP;

sai_trap_attr[0].value=&high_prio_group;

sai_trap_attr[1].id= SAI_HOSTIF_TRAP_ATTR_TRAP_ACTION;

sai_trap_attr[1].value= SAI_PACKET_ACTION_TRAP;

sai_trap_attr[2].id= SAI_HOSTIF_TRAP_ATTR_TRAP_TYPE;

sai_trap_attr[2].value= SAI_HOSTIF_TRAP_TYPE_STP;

sai_create_hostif_trap_fn(&host_trap_id[0],2, sai_trap_attr);

// configure LACP trap-id

sai_trap_attr[0].id=SAI_HOSTIF_TRAP_ATTR_TRAP_GROUP;

sai_trap_attr[0].value=&high_prio_group;

sai_trap_attr[1].id= SAI_HOSTIF_TRAP_ATTR_TRAP_ACTION;

sai_trap_attr[1].value=SAI_PACKET_ACTION_TRAP;

sai_trap_attr[2].id= SAI_HOSTIF_TRAP_ATTR_TRAP_TYPE;

sai_trap_attr[2].value= SAI_HOSTIF_TRAP_TYPE_LACP;

sai_create_hostif_trap_fn(&host_trap_id[1], 2, sai_trap_attr);

// configure ARP trap-id

sai_trap_attr[0].id=SAI_HOSTIF_TRAP_ATTR_TRAP_GROUP;

sai_trap_attr[0].value=&med_prio_group;

sai_trap_attr[1].id= SAI_HOSTIF_TRAP_ATTR_TRAP_ACTION;

sai_trap_attr[1].value= SAI_PACKET_ACTION_LOG;

sai_trap_attr[2].id= SAI_HOSTIF_TRAP_ATTR_TRAP_TYPE;

sai_trap_attr[2].value= SAI_HOSTIF_TRAP_TYPE_ARP_REQUEST;

sai_create_hostif_trap_fn(&host_trap_id[2], 2, sai_trap_attr);

…

//configure router exception

sai_trap_attr[0].id= SAI_HOSTIF_TRAP_ATTR_TRAP_GROUP;

sai_trap_attr[0].value=&low_prio_group;

sai_trap_attr[1].id= SAI_HOSTIF_TRAP_ATTR_TRAP_ACTION;

sai_trap_attr[1].value= SAI_PACKET_ACTION_TRAP;

sai_trap_attr[2].id= SAI_HOSTIF_TRAP_ATTR_TRAP_TYPE;

sai_trap_attr[2].value= SAI_HOSTIF_TRAP_TYPE_ROUTER_MIN;

sai_create_hostif_trap_fn(&host_trap_id[3], 2, sai_trap_attr);

### Step 4- configure the host table

sai_object_id_t host_table_entry[4];

sai_attribute_t sai_if_channel_attr[3];

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_TRAP_ID;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_TRAP_ID;

sai_if_channel_attr[1].value=host_trap_id[0]; // Object referencing STP trap

sai_if_channel_attr[2].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[2].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_NETDEV_LOGICAL_PORT;

sai_create_hostif_table_entry_fn(&host_table_entry[0], 3, sai_if_channel_attr);

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_TRAP_ID;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_TRAP_ID;

sai_if_channel_attr[1].value=host_trap_id[1]; // Object referencing LACP trap

sai_if_channel_attr[2].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[2].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_NETDEV_PHYSICAL_PORT;

sai_create_hostif_table_entry_fn(&host_table_entry[1], 3, sai_if_channel_attr);

…

## Configuration example – using file descriptor channel

Same setup and packet types as above example working with a single file descriptor.

### Step 1- create three trap groups

Same as 3.5.1

### Step 2- create host interface FD

Creation of the file descriptor (the same logic and flow can be used in order to create more than one file descriptor)

sai_object_id_t host_if_id;

sai_attribute_t sai_if_channel_attr[1];

sai_if_channel_attr[0].id=SAI_HOST_INTERFACE_ATTR_TYPE;

sai_if_channel_attr[0].value=SAI_HOST_INTERFACE_TYPE_FD;

sai_create_host_interface_fn(&host_if_id, 1, sai_if_channel_attr);

### Step 3- configure the trap-ids

Same as 3.5.3

### Step 4- configure the host table

sai_object_id_t host_table_entry[4];

sai_attribute_t sai_if_channel_attr[4];

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_TRAP_ID;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_TRAP_ID;

sai_if_channel_attr[1].value=host_trap_id[0]; // Object referencing STP trap

sai_if_channel_attr[2].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[2].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_FD;

sai_if_channel_attr[3].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_HOST_IF;

sai_if_channel_attr[3].value=host_if_id; // host interface of type file descriptor

sai_create_hostif_table_entry_fn(&host_table_entry[0], 4, sai_if_channel_attr);

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_TRAP_ID;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_TRAP_ID;

sai_if_channel_attr[1].value=host_trap_id[1]; // Object referencing LACP trap

sai_if_channel_attr[2].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[2].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_FD;

sai_if_channel_attr[3].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_HOST_IF;

sai_if_channel_attr[3].value=host_if_id; // host interface of type file descriptor

sai_create_hostif_table_entry_fn(&host_table_entry[1], 3, sai_if_channel_attr);

…

### Step 5- send and receive

// receive flow

void *buffer;

sai_size_t buffer_size,

sai_attribute_t sai_packet_attr[3];

uint32_t attr_count = 3;

// alloocate buffer

buffer_alloc(&buffer, CONTROL_MTU);

while(TRUE)

{

buffer_size = CONTROL_MTU;

sai_recv_hostif_packet_fn(host_if_id, buffer, &buffer_size, &attr_count, sai_packet_attr);

handle_packet(buffer, buffer_size, attr_count, sai_packet_attr);

);

// send flow

void *buffer;

sai_attribute_t sai_packet_send_attr[3];

// send STP packet through port 4

buffer_alloc(&buffer, STP_FRAME_SIZE);

stp_buffer_set(buffer);

sai_packet_send_attr[0].id= SAI_PACKET_EGRESS_PORT;

sai_packet_send_attr[0].value=4;

sai_packet_send_attr[1].id= SAI_PACKET_EGRESS_QUEUE;

sai_packet_send_attr[1].value= queue_id; //queue_id is a queue element created via QoS SAI;

sai_packet_send_attr[2].id= SAI_PACKET_TX_TYPE;

sai_packet_send_attr[2].value= SAI_TX_TYPE_PIPELINE_BYPASS;

sai_send_hostif_packet_fn(host_if_id, buffer, STP_FRAME_SIZE, 3, sai_packet_send_attr);

## Configuration example – using packet receive callback channel

Same setup and packet types as above example working with a packet receive callback

### Step 1- create three trap groups

Same as 3.5.1

### Step 2- configure the trap-ids

Same as 3.5.2

### Step 3- configure the host table

sai_object_id_t host_table_entry[4];

sai_attribute_t sai_if_channel_attr[3];

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_TRAP_ID;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_TRAP_ID;

sai_if_channel_attr[1].value=host_trap_id[0]; // Object referencing STP trap

sai_if_channel_attr[2].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[2].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_CB;

sai_create_hostif_table_entry_fn(&host_table_entry[0], 3, sai_if_channel_attr);

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_TRAP_ID;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_TRAP_ID;

sai_if_channel_attr[1].value=host_trap_id[1]; // Object referencing LACP trap

sai_if_channel_attr[2].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[2].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_CB;

sai_create_hostif_table_entry_fn(&host_table_entry[1], 3, sai_if_channel_attr);

…

### Step 4- send and receive

// recive flow SAI via CB

SAI calls application callback on_packet_event(

_In_ const void *buffer,

_In_ sai_size_t buffer_size,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

)

// send flow

void * buffer;

sai_attribute_t sai_packet_send_attr[3];

// send STP packet through port 4

buffer_alloc(&buffer, STP_FRAME_SIZE);

stp_buffer_set(buffer);

sai_packet_send_attr[0].id= SAI_PACKET_EGRESS_PORT;

sai_packet_send_attr[0].value=4;

sai_packet_send_attr[1].id= SAI_PACKET_EGRESS_QUEUE;

sai_packet_send_attr[1].value= queue_id; //queue_id is a queue element created via QoS SAI;

sai_packet_send_attr[2].id= SAI_PACKET_TX_TYPE;

sai_packet_send_attr[2].value= SAI_TX_TYPE_PIPELINE_BYPASS;

sai_send_hostif_packet_fn(0, buffer, STP_FRAME_SIZE, 3 ,sai_packet_send_attr);

## Configuration example – trap via ACL

In the below example we will configure a system running using ACL to trap packet to the CPU

The user channel will be set globally (not per trap-id )  to OS net device

### Step 1- configure the trap id

Same as 3.5.1

### Step 2- create host interface netdev

Same as 3.5.2

### Step 3- configuring the host table

sai_object_id_t host_table_entry[4];

sai_attribute_t sai_if_channel_attr[2];

sai_if_channel_attr[0].id=SAI_HOSTIF_TABLE_ENTRY_ATTR_TYPE;

sai_if_channel_attr[0].value= SAI_HOST_INTERFACE_TABLE_ENTRY_TYPE_WILDCARD;

sai_if_channel_attr[1].id= SAI_HOSTIF_TABLE_ENTRY_ATTR_CHANNEL;

sai_if_channel_attr[1].value=SAI_HOST_INTERFACE_TABLE_ENTRY_CHANNEL_TYPE_PHYSICAL_NETDEV;

sai_create_hostif_table_entry_fn(&host_table_entry[0], 2, sai_if_channel_attr);

### Step 2- ACL configuration

- Create ACL table

  - Set ACL table priority attribute

- Create an ACL entry

  - Set ACL entry match fields

  - Set ACL entry priority

    - Set ACL entry action fields

      - SAI_ACL_ENTRY_ATTR_ACTION_TRAP_TO_CPU to true

      - SAI_ACL_ENTRY_ATTR_ACTION_SET_POLICER with pre define policer

      - SAI_ACL_ENTRY_ATTR_ACTION_SET_COS with pre define traffic class

  - Bing  ACL entry to a ACL table